# VRX Boat Thruster Configuration Summary

Platform: VRX Simulation (Gazebo)  |  Reference Frame: Body-fixed, origin at Center of Mass (C)

## 1. Coordinate Convention

+X: Bow (forward)

-X: Stern (backward)

+Y: Port (left)

-Y: Starboard (right)

+Z: Up

-Z: Down

Origin (0,0,0): Center of Mass (C)

## 2. Master Thruster Configuration

| Property | T₁ (Port Bow) | T₂ (Starboard Bow) | T₃ (Stern Propeller) |
| --- | --- | --- | --- |
| Hardware | APISQUEEN U92 Pro 24V | APISQUEEN U92 Pro 24V | Cruise 3.0 RS 48V |
| Type | Underwater Thruster | Underwater Thruster | Azimuthing Propeller |
| X Position | +1.03 m | +1.03 m | -1.50 m |
| Y Position | +0.74 m | -0.74 m | 0.00 m |
| Z Position | -0.23 m | -0.23 m | -0.25 m |
| Facing Axis | +Y | -Y | -X |
| Fwd Rotation Thrust | -Y (→ Starboard) | +Y (→ Port) | +X (→ Forward) |
| Rev Rotation Thrust | +Y (→ Port) | -Y (→ Starboard) | -X (→ Backward) |
| Axis of Rotation | Fixed (none) | Fixed (none) | Z-axis (yaw) |
| Rotation Range | None | None | ±21° (total 42°) |
| Shaft Length | — | — | 0.40 m |
| Primary Function | Sway / Yaw control | Sway / Yaw control | Surge + Yaw control |
| Voltage | 24V | 24V | 48V |

## 3. Degrees of Freedom Controlled

| DOF | Controlled By | How |
| --- | --- | --- |
| Surge (+X forward) | T₃ forward rotation | Thrust in +X direction pushes boat forward |
| Surge (-X backward) | T₃ reverse rotation | Thrust in -X direction pushes boat backward |
| Sway (Y) | T₁ + T₂ differential | Both spin same direction = pure sway |
| Yaw (Z rotation) | T₃ azimuth ±21° + T₁/T₂ differential | T₁/T₂ opposite = spin in place |
| Heave / Roll / Pitch | Not directly controlled | — |

## 4. Gazebo SDF Pose Reference (X Y Z Roll Pitch Yaw)

| Thruster | Pose (X Y Z R P Y) | Yaw Reasoning |
| --- | --- | --- |
| T₁ Port Bow | 1.03 0.74 -0.23 0 0 -1.5708 | Faces +Y axis (90° CCW) |
| T₂ Starboard Bow | 1.03 -0.74 -0.23 0 0 1.5708 | Faces -Y axis (90° CW) |
| T₃ Stern Propeller | -1.50 0 -0.25 0 0 3.1416 | Faces -X axis (180°) |

Note: Yaw values in radians. T₁/T₂ oriented to thrust along Y-axis; T₃ oriented to face -X (stern) direction for forward propulsion.